# CPE403 – Advanced Embedded Systems

# Design Assignment 2

Name: Jenifer Christina

Email: chrisj14@unlv.nevada.edu

Github Repository link: https://github.com/chrisj14/CCS-Assignment

Youtube Playlist link: https://youtu.be/cwDEEj2b8To

Follow the submission guideline to be awarded points for this Assignment.

- Code for Tasks. for each task submit the modified or included code (from the base code) with highlights and justifications of the modifications. Also include the comments. If no base code is provided, submit the base code for the first task only. Use separate page for each task.

Task 01: Interface the TMP006 IR Temperature Sensor (MKII Educational Boosterpack). Determine the object temperature. Perform all computations using IQ Math Structures and Functions. Display the results in the terminal.

#include <stdint.h>

#include <stdbool.h>

#include <string.h>

#include "inc/hw_ints.h"

#include "inc/tm4c123gh6pm.h"   //def. for the interrupt and register assignments on the Tiva C Series device on the launchPad board

#include "inc/hw_memmap.h"

#include "inc/hw_types.h"

#include "inc/hw_uart.h"

#include "driverlib/fpu.h"

#include "driverlib/gpio.h"

#include "driverlib/interrupt.h"

#include "driverlib/pin_map.h"

#include "driverlib/rom.h"

#include "driverlib/sysctl.h"

#include "driverlib/udma.h"

#include "driverlib/timer.h"    //Defines and macros for Timer API of driverLib.

#include "driverlib/adc.h"

#include "driverlib/debug.h"

#include "driverlib/uart.h"

#include "utils/uartstdio.h"

// For Temperature value

uint32_t ui32Period;

char buffer [4];

uint32_t ui32ADC0Value[4];

volatile uint32_t ui32TempAvg;

volatile uint32_t ui32TempValueC;

volatile uint32_t ui32TempValueF;

// Define source and destination buffers

#define MEM_BUFFER_SIZE         1024

static uint32_t g_ui32SrcBuf[MEM_BUFFER_SIZE];

static uint32_t g_ui32DstBuf[MEM_BUFFER_SIZE];

// Define errors counters

static uint32_t g_ui32DMAErrCount = 0;

static uint32_t g_ui32BadISR = 0;

// Define transfer counter

static uint32_t g_ui32MemXferCount = 0;

// The control table used by the uDMA controller.  This table must be aligned to a 1024 byte boundary.

#pragma DATA_ALIGN(pui8ControlTable, 1024)

uint8_t pui8ControlTable[1024];

// Library error routine

#ifdef DEBUG

void

__error__(char *pcFilename, uint32_t ui32Line)

{

}

#endif

// uDMA transfer error handler

void

uDMAErrorHandler(void)

{

uint32_t ui32Status;

// Check for uDMA error bit

ui32Status = uDMAErrorStatusGet();

// If there is a uDMA error, then clear the error and increment the error counter.

if(ui32Status)

{

uDMAErrorStatusClear();

g_ui32DMAErrCount++;

}

}

// uDMA interrupt handler. Run when transfer is complete.

void

uDMAIntHandler(void)

{

uint32_t ui32Mode;

// Check for the primary control structure to indicate complete.

ui32Mode = uDMAChannelModeGet(UDMA_CHANNEL_SW);

if(ui32Mode == UDMA_MODE_STOP)

{

// Increment the count of completed transfers.

g_ui32MemXferCount++;

// Configure it for another transfer.

uDMAChannelTransferSet(UDMA_CHANNEL_SW, UDMA_MODE_AUTO,

g_ui32SrcBuf, g_ui32DstBuf, MEM_BUFFER_SIZE);

// Initiate another transfer.

uDMAChannelEnable(UDMA_CHANNEL_SW);

uDMAChannelRequest(UDMA_CHANNEL_SW);

uint32_t ui32Status;

ui32Status = UARTIntStatus(UART0_BASE, true); //get interrupt status

UARTIntClear(UART0_BASE, ui32Status); //clear the asserted interrupts

while(UARTCharsAvail(UART0_BASE)) //loop while there are chars

{

char cChar=UARTCharGet(UART0_BASE);

UARTCharPutNonBlocking(UART0_BASE, cChar); //echo character

if (cChar=='R') {       //Turn on RED LED

GPIOPinWrite(GPIO_PORTF_BASE, GPIO_PIN_1, GPIO_PIN_1);

}

else if (cChar=='r') {  //Turn off RED LED

GPIOPinWrite(GPIO_PORTF_BASE, GPIO_PIN_1, 0);

}

else if (cChar=='G') {  //Turn on Green LED

GPIOPinWrite(GPIO_PORTF_BASE, GPIO_PIN_3, GPIO_PIN_3);

}

else if (cChar=='g') {  //Turn off Green LED

GPIOPinWrite(GPIO_PORTF_BASE, GPIO_PIN_3, 0);

}

else if (cChar=='B') {  //Turn on Blue LED

GPIOPinWrite(GPIO_PORTF_BASE, GPIO_PIN_2, GPIO_PIN_2);

}

else if (cChar=='b') {  //Turn off Blue LED

GPIOPinWrite(GPIO_PORTF_BASE, GPIO_PIN_2, 0);

}

else if (cChar=='T') {  //Show Temperature in Centigrade

ADCIntClear(ADC0_BASE,2);

ADCProcessorTrigger(ADC0_BASE, 2);

ADCSequenceDataGet(ADC0_BASE, 2, ui32ADC0Value);

ui32TempAvg = (ui32ADC0Value[0] + ui32ADC0Value[1] + ui32ADC0Value[2] + ui32ADC0Value[3] + 2)/4;

ui32TempValueC = (1475 - ((2475 * ui32TempAvg)) / 4096)/10;

ui32TempValueF = ((ui32TempValueC * 9) + 160) / 5;

UARTprintf("\n C %3d\t \n",ui32TempValueC );

}

else if (cChar=='t') {  //Show Temperature in Farenheit

ADCIntClear(ADC0_BASE,2);

ADCProcessorTrigger(ADC0_BASE, 2);

ADCSequenceDataGet(ADC0_BASE, 2, ui32ADC0Value);

ui32TempAvg = (ui32ADC0Value[0] + ui32ADC0Value[1] + ui32ADC0Value[2] + ui32ADC0Value[3] + 2)/4;

ui32TempValueC = (1475 - ((2475 * ui32TempAvg)) / 4096)/10;

ui32TempValueF = ((ui32TempValueC * 9) + 160) / 5;

UARTprintf("\n F %3d\t \n",ui32TempValueF );

}

else if (cChar=='S') {  //Show LED Status

UARTprintf("\n");

if (GPIOPinRead(GPIO_PORTF_BASE, GPIO_PIN_1))

UARTprintf("Red LED is on \n");

if (GPIOPinRead(GPIO_PORTF_BASE, GPIO_PIN_2))

UARTprintf("Blue LED is on \n");

if (GPIOPinRead(GPIO_PORTF_BASE, GPIO_PIN_3))

UARTprintf("Green LED is on \n");

}

}

}

// If the channel is not stopped, then something is wrong.

else

{

g_ui32BadISR++;

}

}

// Initialize the uDMA software channel to perform a memory to memory uDMA transfer.

void

InitSWTransfer(void)

{

uint32_t ui32Idx;

// Fill the source memory buffer with a simple incrementing pattern.

for(ui32Idx = 0; ui32Idx < MEM_BUFFER_SIZE; ui32Idx++)

{

g_ui32SrcBuf[ui32Idx] = ui32Idx;

}

// Enable interrupts from the uDMA software channel.

IntEnable(INT_UDMA);

// Place the uDMA channel attributes in a known state. These should already be disabled by default.

uDMAChannelAttributeDisable(UDMA_CHANNEL_SW,

UDMA_ATTR_USEBURST | UDMA_ATTR_ALTSELECT |

(UDMA_ATTR_HIGH_PRIORITY |

UDMA_ATTR_REQMASK));

// Configure the control parameters for the SW channel.  The SW channel

// will be used to transfer between two memory buffers, 32 bits at a time,

// and the address increment is 32 bits for both source and destination.

// The arbitration size will be set to 8, which causes the uDMA controller

// to rearbitrate after 8 items are transferred.  This keeps this channel from

// hogging the uDMA controller once the transfer is started, and allows other

// channels to get serviced if they are higher priority.

uDMAChannelControlSet(UDMA_CHANNEL_SW | UDMA_PRI_SELECT,

UDMA_SIZE_32 | UDMA_SRC_INC_32 | UDMA_DST_INC_32 |

UDMA_ARB_8);

// Set up the transfer parameters for the software channel.  This will

// configure the transfer buffers and the transfer size.  Auto mode must be

// used for software transfers.

uDMAChannelTransferSet(UDMA_CHANNEL_SW | UDMA_PRI_SELECT,

UDMA_MODE_AUTO, g_ui32SrcBuf, g_ui32DstBuf,

MEM_BUFFER_SIZE);

// Now the software channel is primed to start a transfer.  The channel

// must be enabled.  For software based transfers, a request must be

// issued.  After this, the uDMA memory transfer begins.

uDMAChannelEnable(UDMA_CHANNEL_SW);

uDMAChannelRequest(UDMA_CHANNEL_SW);

}

int

main(void)

{

//Configure peripherals

SysCtlPeripheralEnable(SYSCTL_PERIPH_GPIOA);

SysCtlPeripheralEnable(SYSCTL_PERIPH_GPIOF);

SysCtlPeripheralEnable(SYSCTL_PERIPH_ADC0);

//Setup for ADC

ADCHardwareOversampleConfigure(ADC0_BASE, 32);

ADCSequenceConfigure(ADC0_BASE, 2, ADC_TRIGGER_PROCESSOR, 0);

ADCSequenceStepConfigure(ADC0_BASE, 2, 0, ADC_CTL_TS);

ADCSequenceStepConfigure(ADC0_BASE, 2, 1, ADC_CTL_TS);

ADCSequenceStepConfigure(ADC0_BASE, 2, 2, ADC_CTL_TS);

ADCSequenceStepConfigure(ADC0_BASE, 2, 3, ADC_CTL_TS|ADC_CTL_IE|ADC_CTL_END);

ADCSequenceEnable(ADC0_BASE, 2);

GPIOPinTypeGPIOOutput(GPIO_PORTF_BASE, GPIO_PIN_1|GPIO_PIN_2|GPIO_PIN_3); //enable pin for LED PF2

IntMasterEnable(); //enable processor interrupts

FPULazyStackingEnable();

SysCtlClockSet(SYSCTL_SYSDIV_4 | SYSCTL_USE_PLL | SYSCTL_OSC_MAIN |

SYSCTL_XTAL_16MHZ);

SysCtlPeripheralClockGating(true);

SysCtlPeripheralEnable(SYSCTL_PERIPH_UDMA);

SysCtlPeripheralSleepEnable(SYSCTL_PERIPH_UDMA);

IntEnable(INT_UDMAERR);

uDMAEnable();

uDMAControlBaseSet(pui8ControlTable);

InitSWTransfer();

UARTprintf("Enter the cmd: \n"

"R: Red LED, \n"

"G: Green LED, \n"

"B: Blue LED, \n"

"T: Temperature, \n"

"S: status of the LEDs. \n");

while(1)

{

}

}

- Block diagram and/or Schematics showing the components, pins used, and interface.

Boosterpack is placed on top of Launchpad.

Connected to USB

Temperature Sensor TMP006

- Screenshots of the IDE, physical setup, debugging process - Provide screenshot of successful compilation, screenshots of registers, variables, graphs, etc.

- Declaration

I understand the Student Academic Misconduct Policy - http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Jenifer Christina